一、输入按键映射（GPIO端口）

| 按键 | 对应位 | GPIO口 | 描述 |
| --- | --- | --- | --- |
| UP | 0 | PB0 | 方向上 |
| DOWN | 1 | PB1 | 方向下 |
| LEFT | 2 | PB2 | 方向左 |
| RIGHT | 3 | PB3 | 方向右 |
| LP | 4 | PB4 | 轻拳 |
| MP | 5 | PB5 | 中拳 |
| HP | 6 | PB6 | 重拳 |
| LK | 7 | PB7 | 轻脚 |
| MK | 8 | PB8 | 中脚 |
| HK | 9 | PB9 | 重脚 |
| SUPER | 10 | PB10 | 超级技按键 |
| SP1 | 11 | PB11 | 特殊技1 |
| SP2 | 12 | PB12 | 特殊技2 |
| SP3 | 13 | PB13 | 特殊技3 |
| SP4 | 14 | PB14 | 特殊技4 |
| X | 15 | PB15 | 镜像翻转按键 |
| COMBO1 | 16 | PA10 | 连招组合1 |
| COMBO2 | 17 | PA11 | 连招组合2 |
| COMBO3 | 18 | PA12 | 连招组合3 |

二、输出按键映射（PA0~PA9 开漏输出）

| 输出按键 | 对应位 | GPIO口 | 描述 |
| --- | --- | --- | --- |
| OUT_UP | 0 | PA0 | 输出方向上 |
| OUT_DOWN | 1 | PA1 | 输出方向下 |
| OUT_LEFT | 2 | PA2 | 输出方向左 |
| OUT_RIGHT | 3 | PA3 | 输出方向右 |
| OUT_LP | 4 | PA4 | 输出轻拳 |
| OUT_MP | 5 | PA5 | 输出中拳 |
| OUT_HP | 6 | PA6 | 输出重拳 |
| OUT_LK | 7 | PA7 | 输出轻脚 |
| OUT_MK | 8 | PA8 | 输出中脚 |
| OUT_HK | 9 | PA9 | 输出重脚 |

开漏模式：PA0~PA9 输出按键拉低表示按下，默认高

输入按键：PB0~PB15、PA10~PA12 机械按键一端接 IO 另一端接地

#include "stm32f10x.h"

#include <stdint.h>

typedef uint32_t IN_STATE;

typedef uint16_t OUT_STATE;

volatile IN_STATE debug_in = 0;

// ???

#define IN_UP         (1U << 0)

#define IN_DOWN       (1U << 1)

//#define IN_LEFT       (1U << 2)

#define IN_RIGHT      (1U << 3)

// ???

#define IN_LP         (1U << 4)

#define IN_MP         (1U << 5)

#define IN_HP         (1U << 6)

#define IN_LK         (1U << 7)

#define IN_MK         (1U << 8)

#define IN_HK         (1U << 9)

// ???

#define IN_SUPER      (1U << 10)

// SP ??

#define IN_SP1        (1U << 11)

#define IN_SP2        (1U << 12)

#define IN_SP3        (1U << 13)

#define IN_SP4        (1U << 14)

// COMBO ??

#define IN_COMBO1     (1U << 16)

#define IN_COMBO2     (1U << 17)

#define IN_COMBO3     (1U << 18)

//#define IN_LA1        (1u << 19)

// X ???

#define IN_X          (1U << 15)

#define OUT_UP        (1U << 0)

#define OUT_DOWN      (1U << 1)

#define OUT_LEFT      (1U << 2)

#define OUT_RIGHT     (1U << 3)

#define OUT_LP        (1U << 4)

#define OUT_MP        (1U << 5)

#define OUT_HP        (1U << 6)

#define OUT_LK        (1U << 7)

#define OUT_MK        (1U << 8)

#define OUT_HK        (1U << 9)

#define FPS 60

volatile uint8_t tick_flag = 0;

void SysTick_Init(void)

{

// STM32F103C8T6 ???? 72 MHz

// SysTick ??????? HCLK = 72 MHz

// ?? = 1/60 ? = 16.6667 ms

// SysTick ???? = 72 MHz * 0.0166667 ˜ 1200000

SysTick->LOAD  = 1200000 - 1; // ????

SysTick->VAL   = 0;           // ?????

SysTick->CTRL  = SysTick_CTRL_CLKSOURCE_Msk |  // HCLK

SysTick_CTRL_TICKINT_Msk   |  // ????

SysTick_CTRL_ENABLE_Msk;      // ??

}

// SysTick ??????

void SysTick_Handler(void)

{

tick_flag = 1; // ????

}

void GPIO_Init_All(void)

{

RCC->APB2ENR |= RCC_APB2ENR_AFIOEN;

// ?? JTAG,?? SWD

AFIO->MAPR |= AFIO_MAPR_SWJ_CFG_1;

// ??? GPIO ??

RCC->APB2ENR |= RCC_APB2ENR_IOPAEN;

RCC->APB2ENR |= RCC_APB2ENR_IOPBEN;

GPIOA->CRL &= ~(0xFFFFFFFF);   // ??

GPIOA->CRL |= 0x77777777;      // ??????,50MHz

// PA8~PA9

GPIOA->CRH &= ~(0xFF);         // ?? PA8~PA9 ??

GPIOA->CRH |= 0x77;            // ??????,50MHz

// ??????

GPIOA->ODR |= 0x03FF;          // PA0~PA9 ???(???)

//************ ?? PB0~PB15(????) ************

// PB0~PB7

GPIOB->CRL &= ~(0xFFFFFFFF);

GPIOB->CRL |= 0x88888888;      // ????

// PB8~PB15

GPIOB->CRH &= ~(0xFFFFFFFF);

GPIOB->CRH |= 0x88888888;      // ????

// ??????

GPIOB->ODR |= 0xFFFF;

// ************ ?? PA10~PA12(????,?????) ************

// PA10~PA12 ? CRH ?

GPIOA->CRH &= ~((0xF << 8) | (0xF << 12) | (0xF << 16));

GPIOA->CRH |=  (0x8 << 8) | (0x8 << 12) | (0x8 << 16);

GPIOA->ODR |=  (1<<10) | (1<<11) | (1<<12);

}

IN_STATE ReadInputs(void)

{

IN_STATE state = 0;

uint32_t pb = ~GPIOB->IDR; // ???1

uint32_t pa = ~GPIOA->IDR;

// PB0~PB15

state |= (pb & 0xFFFF);      // PB0~PB15

// PA10~PA12 -> ??? IN_COMBO1~3 ??? 16~18

state |= ((pa >> 10) & 0x7) << 16;

return state;

}

#define MAX_FRAMES      4

#define HOLD_THRESHOLD  10      // 10?˜167ms(60FPS)

#define QUEUE_LEN       4

// ----------------- ???? -----------------

#define ACT_SP1        1

#define ACT_SP2        2

#define ACT_SP3        3

#define ACT_SP4        4

#define ACT_SUPER      5

#define ACT_SP1_HOLD   11

#define ACT_SP2_HOLD   12

#define ACT_SP3_HOLD   13

#define ACT_SP4_HOLD   14

// ----------------- ???? -----------------

typedef struct {

uint8_t pressed;

uint8_t prev;

uint16_t hold_frames;

} KeyState;

KeyState sp1_key = {0};

KeyState sp2_key = {0};

KeyState sp3_key = {0};

KeyState sp4_key = {0};

KeyState super_key = {0};

// ----------------- ???? -----------------

typedef struct {

uint8_t active;

uint8_t type;

uint8_t frame;

uint8_t dir;     // 0=? 1=? 2=?

} ActionState;

typedef struct {

uint8_t type;

uint8_t dir;

} Action;

Action action_queue[QUEUE_LEN] = {0};

uint8_t queue_head = 0;

uint8_t queue_tail = 0;

ActionState current_action = {0,0,0,0};

static void UpdateKey(KeyState *k, uint8_t is_pressed)

{

k->prev = k->pressed;

k->pressed = is_pressed;

if(k->pressed)

k->hold_frames++;

else

k->hold_frames = 0;

}

void EnqueueAction(uint8_t type, uint8_t dir)

{

uint8_t next = (queue_tail + 1) % QUEUE_LEN;

if(next != queue_head)   // ????

{

action_queue[queue_tail].type = type;

action_queue[queue_tail].dir  = dir;

queue_tail = next;

}

}

static void MirrorDirection(uint32_t *left, uint32_t *right, uint8_t mirror)

{

if(mirror)

{

uint32_t tmp = *left;

*left  = *right;

*right = tmp;

}

}

int test_flag=10;

OUT_STATE Logic_Process(IN_STATE in)

{

OUT_STATE out = 0;

uint8_t mirror = (in & IN_X) ? 1 : 0;

// ----------------- ?????? -----------------

UpdateKey(&sp1_key, (in & IN_COMBO1) ? 1 : 0);

//UpdateKey(&sp2_key, (in & IN_SP2) ? 1 : 0);

UpdateKey(&sp3_key, (in & IN_SP3) ? 1 : 0);

//UpdateKey(&sp4_key, (in & IN_COMBO1) ? 1 : 0);

UpdateKey(&super_key, (in & IN_SP1) ? 1 : 0);

// ----------------- ???? -----------------

uint8_t dir = 3;

if(in & IN_DOWN) dir = 0;

else if(in & IN_SUPER) dir = 1;

else if(in & IN_RIGHT) dir = 2;

if(in & IN_SP1)

{

test_flag = 1;

}

else

{

test_flag = 0;

}

// ----------------- ????? -----------------

if(!sp1_key.prev && sp1_key.pressed)

EnqueueAction(

(sp1_key.hold_frames >= HOLD_THRESHOLD) ? ACT_SP1_HOLD : ACT_SP1,

dir

);

if(!sp2_key.prev && sp2_key.pressed)

EnqueueAction(

(sp2_key.hold_frames >= HOLD_THRESHOLD) ? ACT_SP2_HOLD : ACT_SP2,

dir

);

if(!sp3_key.prev && sp3_key.pressed)

EnqueueAction(

(sp3_key.hold_frames >= HOLD_THRESHOLD) ? ACT_SP3_HOLD : ACT_SP3,

dir

);

if(!sp4_key.prev && sp4_key.pressed)

EnqueueAction(

(sp4_key.hold_frames >= HOLD_THRESHOLD) ? ACT_SP4_HOLD : ACT_SP4,

dir

);

if(!super_key.prev && super_key.pressed)

EnqueueAction(ACT_SUPER, dir);

// ----------------- ????? -----------------

if(!current_action.active && queue_head != queue_tail)

{

current_action.active = 1;

current_action.type   = action_queue[queue_head].type;

current_action.dir    = action_queue[queue_head].dir;

current_action.frame  = 0;

queue_head = (queue_head + 1) % QUEUE_LEN;

}

// ----------------- ???? -----------------

if(current_action.active)

{

uint32_t down  = OUT_DOWN;

uint32_t left  = OUT_LEFT;

uint32_t right = OUT_RIGHT;

MirrorDirection(&left, &right, mirror);

switch(current_action.type)

{

// ================= SP NORMAL =================

// ================= SP1 =================

case ACT_SP1:

{

switch(current_action.frame)

{

case 0: out |= down; break;

case 1: out |= right; break;

case 2: out |= left; break;

case 3: out |= OUT_UP;break;

}

}

break;

// ================= SP2 =================

case ACT_SP2:

{

switch(current_action.frame)

{

case 0: out |= right; break;

case 1: out |= down; break;

case 2: out |= (down | right);break;

case 3:

switch(current_action.dir)

{

case 0: out |= OUT_MK; break;

case 1: out |= OUT_LK; break;

case 2: out |= OUT_MK | OUT_HK;break;

default: out |= OUT_HK; break;

}

break;

}

}

break;

// ================= SP3 =================

case ACT_SP3:

{

switch(current_action.frame)

{

case 0: out |= down; break;

case 1: out |= (down | left); break;

case 2: out |= left; break;

case 3:

switch(current_action.dir)

{

case 0: out |= OUT_MK; break;

case 1: out |= OUT_LK; break;

case 2: out |= OUT_HK; break;

default: out |= OUT_MK | OUT_HK; break;

}

break;

}

}

break;

// ================= SP4 =================

case ACT_SP4:

{

switch(current_action.frame)

{

case 0: out |= right; break;

case 1: out |= (OUT_MK | OUT_MP); break;

case 2: out |= right; break;

case 3: out|= right;break;

}

}

break;

// ================= SP HOLD ?? =================

case ACT_SP1_HOLD:

case ACT_SP2_HOLD:

case ACT_SP3_HOLD:

case ACT_SP4_HOLD:

{

uint8_t reverse = (current_action.type==ACT_SP2_HOLD || current_action.type==ACT_SP4_HOLD);

switch(current_action.frame)

{

case 0: out |= down; break;

case 1: out |= reverse ? (down|right):(down|left); break;

case 2: out |= reverse ? right:left; break;

case 3:

switch(current_action.dir)

{

case 0: out |= OUT_MP|OUT_MK; break;

case 1: out |= OUT_LP|OUT_LK; break;

case 2: out |= OUT_HP|OUT_HK; break;

default: out |= OUT_MP|OUT_HP|OUT_MK; break;

}

break;

}

break;

}

// ================= SUPER =================

case ACT_SUPER:

{

// ? 0~4 ?????

if(current_action.frame <= 7)

{

switch(current_action.dir)

{

case 0:

{

switch(current_action.frame)

{

case 0: out |= down; break;

case 1: out |= right; break;

case 2: out |= left; break;

case 3: out |= OUT_UP; break;

case 4: out |= right; break;

case 5: out |= left; break;

case 6: out |= down; break;

case 7: out |= OUT_HP; break;

}

}

break;

case 2:

{

switch(current_action.frame)

{

case 0: out |= down; break;

case 1: out |= (down | right); break;

case 2: out |= right; break;

case 3: out |= down; break;

case 4: out |= (down | right); break;

case 5: out |= right; break;

case 6: out |= OUT_HP; break;

}

}

break;

case 1:

{

switch(current_action.frame)

{

case 0: out |= down; break;

case 1: out |= (down | right); break;

case 2: out |= right; break;

case 3: out |= down; break;

case 4: out |= (down | right); break;

case 5: out |= right; break;

case 6: out |= OUT_HK; break;

}

}

break;

}

}

}

break;

}

current_action.frame++;

// -------- ???? --------

if(current_action.type>=1 && current_action.type<=4 && current_action.frame>=MAX_FRAMES)

{

current_action.active = 0;

current_action.frame  = 0;

}

else if(current_action.type>=11 && current_action.type<=14 && current_action.frame>=MAX_FRAMES)

{

current_action.active = 0;

current_action.frame  = 0;

}

else if(current_action.type==ACT_SUPER && current_action.frame>=8)

{

current_action.active = 0;

current_action.frame  = 0;

}

return out;

}

// ----------------- ?????? -----------------

if(in & IN_SP2)    out |= OUT_UP;

if(in & IN_DOWN)  out |= OUT_DOWN;

if(in & IN_SUPER)  out |= OUT_LEFT;

if(in & IN_RIGHT) out |= OUT_RIGHT;

if(in & IN_LP) out |= OUT_LP;

if(in & IN_MP) out |= OUT_MP;

if(in & IN_HP) out |= OUT_HP;

if(in & IN_LK) out |= OUT_LK;

if(in & IN_MK) out |= OUT_MK;

if(in & IN_HK) out |= OUT_HK;

//if(in & IN_COMBO1) out |= OUT_MP|OUT_MK;

//if(in & IN_SUPER) out |= OUT_DOWN|OUT_LEFT|OUT_LP;

return out;

}

void Output_Update(OUT_STATE out)

{

GPIOA->ODR |= 0x03FF;          // 0x03FF = PA0~PA9

// ????????????

GPIOA->ODR &= ~(out & 0x03FF); // out ????1 ? ??

}

int main(void)

{

GPIO_Init_All();

SysTick_Init();  // ??? 60FPS SysTick

while(1)

{

debug_in = ReadInputs();

if(tick_flag)  // ????

{

tick_flag = 0;

IN_STATE in  = ReadInputs();

OUT_STATE out = Logic_Process(in);

Output_Update(out);

}

}

}